CALIFORNIA STATE UNIVERSITY, NORTHRIDGE

Edge Detection in FPGA: Sobel Filter Design

A graduate project submitted in partial fulfillment of the requirements

For the degree of Master of Science in Computer Engineering

By

Aaron Guadalupe Funes

December 2026

(Official end of the semester)

The graduate project of Aaron Funes is approved:

_______________________________                                                                ________

Professor Aaron Nanas                                                                                          Date

_______________________________                                                                 ________

Dr. Myung Cho                                                                                                           Date

_______________________________                                                           	   _______

Dr. Shahnam Mirzaei, Chair                                                                                 Date

# Table of Contents

Signature Pageii

List of Tablesiv

List of Figuresv

Abstractvi

Introduction1

Design Implementation4

Results8

References10

# List of Tables

Table 1 : Bit Range Definition	5

# List of Figures

Figure 1 : Original Image	1

Figure 2 : Post-processed image with edge detection applied	1

Figure 3: X and Y Filter	2

Figure 4 : Filter Operation	2

Figure 5: Zybo Block Diagram1	3

Figure 6 : Grey Scale Range	3

Figure 7: Sobel Filter Block Design	4

Figure 8: Sobel Filter State Diagram	4

Figure 9: Sobel X & Y State Machine Diagram	5

Figure 10 : Overflow Logic	6

Figure 11: Convoluted Saturation and Overflow	6

Figure 12 : Sobel Block Diagram	7

Figure 13 : Golden Model	8

Figure 14 : Max Pixel Value check	8

Figure 15 : Simulation Results	8

Figure 16 : Test Log Output	9

# Acronym List

| FPGA | Field-Programmable Gate Array |
| --- | --- |
| PL | Programmable Logic |
| AP SOC | All-Programmable System-On-Chip |
| PS | Processing System |

# Abstract

Edge Detection in FPGA: Sobel Filter Design

By

Aaron Guadalupe Funes

Master of Science in Computer Engineering

Edge detection algorithms are an important image processing technique used in our current world. They are used across many applications, medical devices, surveillance systems, and autonomous systems for example. The Sobel Filter is an edge detection algorithm that applies a 3x3 filter upon a subsection of an image to find any edges that may exist. When performed in software the filter is applied consecutively through the data of the image. With the use of a Field-Programmable Gate Array (FPGA), the filter could be applied in parallel allowing the edge detection process to be sped up. In this project, an image will be loaded to the Zybo Z7-20 development board, where the filter design will exist in the programmable logic (PL), have the filter applied to the image, and output the updated image with the edge detection applied.

# Introduction

Edge detection algorithms are an important image processing technique used in our current world. They are used across many applications, medical devices, surveillance systems, and autonomous systems for example. An example of edge detection is shown in Figure 1, the original image, and Figure 2, the post image-processing image.

Figure 1 : Original Image

Figure 2 : Post-processed image with edge detection applied

The Sobel Filter is an edge detection algorithm that applies a 3x3 filter upon a subsection of an image to find any edges that may exist. The filter is performed by applying an X-direction derivative and a y-direction derivative, as shown in Figure 3.

Figure 3: X and Y Filter

To apply the derivative, weights are multiplied to each pixel location with the upper row being pixels (0,0), (0,1), (0,2), where the designation is (row,column). Once all the weights are applied to each pixel location they are summed providing the associated output value for Gx or Gy.

Once the convoluted values are found for Gx and Gy, the following equation, shown in Figure 4, is performed to get the gradient magnitude at the current position of the filter in the image.

Figure 4 : Filter Operation

The Zybo Z7-20 development board is used for the application of this project. The Zybo Z7 is a All Programmable System-on-Chip (AP SoC) device. It contains a dual-core ARM Cortex-A9 processor with Field Programmable Gate Array (FPGA) logic. The board can be broken down in to two sections, programmable logic and the processing system (PS).

Figure 5: Zybo Block Diagram1

In order to access the PL design for the Sobel filter, Advanced eXtensible Interface (AXI) buses are used to establish communications with PS portion. The AXI buses will be used to read and write to addresses chosen as targets for the inputs and outputs of the PL.

To perform the arithmetic correctly in the firmware logic, signed fixed-point arithmetic is used. Using a grey-scale image each pixel is represented by a uint8, unsigned 8-bit, value. On the grey-scale range, a max value of 255 is white while a min value of zero is black.

Figure 6 : Grey Scale Range

# Design Implementation

The Sobel design was broken down into three submodules, as shown in Figure 7. The Sobel X module handles the x-derivative, the Y module handles the y-derivative, and the Square Root module to finalize the output.

Figure 7: Sobel Filter Block Design

Finite-state machines (FSMs) control how the logic transitions through determined stages. The state machine below in Figure 9, is the state flow for the main Sobel filter module. A start signal is used to initiate the module state machine.

Figure 8: Sobel Filter State Diagram

The state machine below in Figure 9, is the state flow for the Sobel X and Y submodules. The arithmetic logic is broken down in to multiple stages to handle the saturation and overflow checks correctly.

Figure 9: Sobel X & Y State Machine Diagram

The input array subsection for both the X and Y sub modules are nine uint8 values. To perform the convolution operation, the max and min ranges need to be determined for the correct bit range for Gx/Gy. The max range is a pixel value of 255 in all the positive weights and a pixel value of zero in every other weight, gaining a maximum value of 1020. This holds true for the minimum value with the negative weights, where the minimum value is -1020. For this to be stored properly in signed fixed point, a bit length of 11 is used. At the most significant bit (MSB) the weight is -210, the sign bit, giving a full range to -1024 to 1023.

| -210 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSB |  |  |  |  |  |  |  |  |  | LSB |

Table 1 : Bit Range Definition

For the multiplication stage a saturation check is added to keep the output value to minimum value of -510 and a positive value of 510. Once the weights are applied the summation stages are broken up into groups of two for the sign bits to be tracked properly.

Figure 10 : Overflow Logic

After the convoluted values are computed the max output from the squared values is 1,040,400. A saturation and overflow check are applied again in the Verilog code, shown in Figure 11, for square and sum calculations for the square root module.

Figure 11: Convoluted Saturation and Overflow

The square root module is based on the algorithm description described in the ICCD conference paper, A New Non-Restoring Square Root Algorithm and Its VLSI Implementations2, by Yamin Li and Wanming Chu. The initial source code, referenced from verilogcodes.blogspot.com3, posted by user vipin. In this square root design, the algorithm does iterations of add and subtracts and no use of multipliers.

The source code was updated with a state machine flow and start signal logic to initiate the computation.  Once the square root value is calculated, a check determines if the value is greater than the max value of 255.

The file type chosen to hold the image data is bitmap (BMP). BMP can store raw, uncompressed pixel data with no concern for losing quality in the image. To decode a BMP file, the header portion provides the relevant information about the image. The information available is the width, height, byte offset location for where the pixel data is located, and number of bytes per pixel. The baseline C code to read a BMP file is referenced from https://github.com/subrata05/image-processing-in-c/4, written by user Subrata Sarkar. A structure holds the information provided by the header and the pixel values. The pixel values starting address location is assign to a pointer variable that is accessed later to provide the pixel matrix needed for the Sobel operation.

In order for the PL and PS to communicate an AXI IP block is generated from the Sobel Verilog code. The IP block is then instantiated in a block diagram with the other required block designs to communicate with the processor, as show in Figure 12.

Figure 12 : Sobel Block Diagram

# Results

The Verilog code is verified through a self-checking randomized input stimulus testbench. The stimulus is an 8-bit 9x1 array, meant to represent a matrix sample from an image. A golden value is calculated using initial integer values, and the square root value is saved to a real variable. The value is floored down to an 8-bit integer value needed to represent the integer pixel value.

Figure 13 : Golden Model

The integer pixel value is clipped to a maximum value of 255 as that is the max pixel intensity value allowed for a pixel, shown in Figure 14.

Figure 14 : Max Pixel Value check

A snippet of the simulation results are shown below in Figure 15.

Figure 15 : Simulation Results

The address indexing was verified on a local C program to confirm the accessing method. A basic filter to apply the brightness was applied and called as a function to confirm the function method.

Figure 16 : Test Log Output

# References

- Zybo Z7 Reference Manual - Digilent Reference

- https://yamin.cis.k.hosei.ac.jp/papers/ICCD96.pdf

- https://verilogcodes.blogspot.com/2020/12/synthesizable-clocked-square-root.html

- https://github.com/subrata05/image-processing-in-c/